# Siemens S210 Causal Relation Revision Confirmation v6

A01691 and A01782 expert review form

# 审核任务

本确认单只处理两条 pending revise 候选。请刘武专家逐条核对完整原文、候选原因和证据，填写最终结论。建议修订不代表 Gold 结论；在专家确认前，两条记录不得进入 Gold v7 或 FTA。

| 字段 | 填写值 |
| --- | --- |
| 审核专家 | 刘武 |
| 审核日期 |  |
| 当前状态 | pending_expert_revision |
| 批准门禁 | approve + causal + source_to_target + causes + fta_eligible=true |

# 填写规则

只有最终原因、证据定位、因果方向、关系类型和 FTA 资格均可接受时，才填写 approve。

如果仍需修改，填写 revise、最终原因、证据和理由；该记录继续保持 pending。

如果关系不成立，填写 reject 和排除理由；该记录不会进入 Gold。

证据位置必须对应完整原文中的实际语句，不能只引用相邻但不支持候选语义的文本。

# 1 A01691 CR-CAND-V5-007

故障描述：Ti and To unsuitable for PN cycle

当前候选原因：No isochronous PROFINET: The PN clock cycle must be at least 4x the current controller clock cycle.

上一轮结论：revise

上一轮专家意见：非等时 PROFINET 分支明确给出 PN clock cycle 的约束，结合上位 Cause 可确认不满足该约束会形成该告警；但当前候选写成“必须至少为 4 倍”的规范要求，而不是实际违规条件。建议规范化为“PN clock cycle is less than 4× the current controller clock cycle”后再入正式 Gold。

建议原因文本：PN clock cycle is less than 4× the current controller clock cycle

建议证据处理：保留原 Cause 证据，并确认该规范要求应规范化为实际违规条件。

## 当前证据

引用：SIEMENS_S210_2019_A01691:cause:5
位置：447-546
原文：No isochronous PROFINET:
The PN clock cycle must be at least 4x the current controller clock cycle.

## 完整原文

A01691 SI Motion: Ti and To unsuitable for PN cycle
Reaction: NONE
Acknowledge: NONE
Cause: The configured times for PROFINET communication are not permitted and the PN cycle is used as the actual value
acquisition cycle for the safe movement monitoring functions:
Isochronous PROFINET:
The sum of Ti and To is too high for the selected PN cycle. The PN clock cycle should be at least 1 current controller cycle
greater than the sum of Ti and To.
No isochronous PROFINET:
The PN clock cycle must be at least 4x the current controller clock cycle.
Notice:
If this alarm is not observed, then message A01711 or A30711 – with the value 1020 ... 1021 – can sporadically occur.
Remedy: Configure Ti and To low so that they are suitable for the PN cycle or increase the PN cycle time.

## 专家最终填写

| 字段 | 填写值 |
| --- | --- |
| causal_status |  |
| direction |  |
| relation_type |  |
| fta_eligible |  |
| overall_decision |  |
| final_cause_text |  |
| evidence_reference |  |
| evidence_text |  |
| evidence_start |  |
| evidence_end |  |
| expert_reason |  |
| reviewer | 刘武 |
| reviewed_at |  |

# 2 A01782 CR-CAND-V5-021

故障描述：SBT brake test incorrect control

当前候选原因：The brake is not configured in p10202. There is a brake test configuration error.

上一轮结论：revise

上一轮专家意见：当前候选文本是“brake 未在 p10202 配置/存在配置错误”，但本条证据引用实际指向 Bit 0 的“resetting the brake test selection”，候选与证据不一致。完整原文的 Bit 2 确实含有该候选语义，但在修正 evidence reference/offset 并重新核验前不得进入 Gold 或 FTA。

建议原因文本：The brake is not configured in configured p10202. There is a brake test configuration error.

建议证据处理：将证据改指向完整原文 Bit 2，而不是当前 Bit 0 的 resetting the brake test selection。

## 当前证据

引用：SIEMENS_S210_2019_A01782:cause:4
位置：307-378
原文：The safe brake test was canceled by resetting the brake test selection.

建议修正证据（仅供核对）：位置 461-596；原文：The brake is not configured in configured p10202.
There is a brake test configuration error. In this case, alarm A01785 is also output.

## 完整原文

A01782 SBT brake test incorrect control
Reaction: NONE
Acknowledge: NONE
Cause: The brake test was canceled as a result of incorrect control.
Alarm value (r2124, interpret binary):
Alarm value 0:
The brake test was canceled as a result of a fault (brake opening time or brake closing time exceeded).
Bit 0:
The safe brake test was canceled by resetting the brake test selection.
Bit 1:
The safe brake test was canceled by resetting the brake test start.
Bit 2:
The brake is not configured in configured p10202.
There is a brake test configuration error. In this case, alarm A01785 is also output.
Note:
SBT: Safe Brake Test
See also: p10202 (SI Motion SBT brake)
Remedy: - check parameterization of the brake test (p10202).
- check as to whether alarm A01785 is present, and if so, evaluate.
- carry out a safe acknowledgment.
- if required, restart the brake test.

## 专家最终填写

| 字段 | 填写值 |
| --- | --- |
| causal_status |  |
| direction |  |
| relation_type |  |
| fta_eligible |  |
| overall_decision |  |
| final_cause_text |  |
| evidence_reference |  |
| evidence_text |  |
| evidence_start |  |
| evidence_end |  |
| expert_reason |  |
| reviewer | 刘武 |
| reviewed_at |  |

# 汇总确认

| 字段 | 填写值 |
| --- | --- |
| 两条记录是否均已完成复核 |  |
| 可进入 Gold v7 的记录 |  |
| 仍需暂缓的记录 |  |
| 专家补充说明 |  |